TU/CDOE

TEZPUR UNIVERSITY

SEMESTER END EXAMINATION (AUT 21 & SPR 22)

DRE 202: ENERGY EFFICIENCY IN THERMAL UTILITIES

Time: 3 Hours                                                           Total Marks: 70

The figures in the right-hand margin indicate marks for the individual question.

1. Fill in the blanks.	          		                                     1×10=10

- The basic cycle of operation of the gas power plants is known as the __________ (Brayton/Carnot) cycle.

- Density difference of hot and cold water is the driving force for flow in case of __________ (natural/ forced) circulation boilers.

- Boilers operating above 221 bar pressure are known as __________ (critical/ supercritical) boilers.

- In a __________ (topping/ bottoming) cycle cogeneration system the fuel is first used to produce power and then the residual heat is used to generate thermal energy.

- (Lignite/ Anthracite) __________ is the youngest type of coal with lowest carbon content.

- Discharging of water from boiler in order to remove the suspended solids and bottom sludge from the boilers is known as __________ (deaeration/blowdown).

- The temperature at which a liquid fuel stops flowing is known as the __________ (flash/pour) point.

- In heat exchanger analysis, the product of the mass flow rate and the specific heat of a fluid are combined into a single quantity called the __________ (heat capacity rate/ heat flow rate).

- The incombustible part of a fuel is __________ (fixed carbon/ash content).

- (Wet/Dry)__________ steam is the best for industrial process heating.

P.T.O

2. Calculate the efficiency of a boiler given that the mass flow of the feed water	 is 5000 kg/h, mass of coal burned is 450 kg/h, calorific value of coal is 28750 kJ/kg, enthalpy of steam produced is 2750 kJ/kg and exit temperature of feed water 105°C. From the point of view of utilization of a fuel for energy liberation through combustion route, what are the key properties that are to be analysed?                5+5= 10

3. Explain with the help of a diagram the different processes taking place in a steam power plant. How furnaces are classified and what are the characteristics of an efficient furnace?                                                                          5+5=10

4. Compare the different types of cogeneration systems

enlisting their relative merits and demerits.		         10

5. Explain what are insulations and refractories? What is

economic thickness of insulation? How is it calculated?

3+3+4=10

6. Explain the various terms associated with the performance  evaluation of heat exchangers.                                              5+5=10

7. Determine the amount of heat, which should be supplied to 2 kg of water at 25°C to convert it into dry steam at 5 bar. (Given hf = 640.1 kJ/kg and hfg = 2107.4 kJ/kg at 5 bar)		            5

8. A gas consists of 70% propane (C3H8) and 30% butane (C4H10) by volume. Find the stoichiometric air-to-fuel ratio.       5

***