The plant management is planning to convert the existing DG set to gas operated engine. The estimated cost of gas is Rs 20/Sm3. If the energy generation is 3.7 kWh/Sm3, calculate the generation cost per kWh with gas as fuel.

4. Answer the following:                                             6+4+5+5=20

- Explain Demand Side Management and the various techniques used for Demand Side Management. What are the benefits of Demand Side Management to customers and utilities distribution companies?

- What is the significance of overall heat transfer coefficient (U-factor) in building energy consumption?

- Calculate fixed electrical energy consumption for a rolling mill consuming 3,50,000 kWh units of electricity to produce 600 MT of product per month and having specific electrical energy consumption of 500 kWh/MT.

- An investment of Rs. 1.0 lakh is made for a variable speed drive at the beginning of the year, which is also the date of operation. The expected savings over 4 years are Rs. 30,000, Rs. 30,000, Rs. 40,000 and Rs 45,000 respectively. Find out the Net Present Value at the end of the 4th year, if the discount rate is 12%. Also calculate the Internal Rate of Return (IRR).

***

TU/ CODL

TEZPUR UNIVERSITY

SEMESTER END EXAMINATION (AUTUMN) 2019

DRE 201: ENERGY MANAGEMENT AND AUDITING

Time:3 Hours				               Total Marks:70

The figures in the right-hand margin indicate marks for the individual question

*****************************************************

1. Choose the correct option                                              2x10=20

a. Replacement of steam based hot water generation by solar

system is an example of

- matching energy usage to the requirement

- maximizing system efficiency

- energy substitution

- fuel substitution

b. Which instrument is used to monitor O2, CO in flue gas?

- Combustion analyzer

- Power analyzer

- Pyrometer

- Fyritte

c. For a cement plant the parameter, ‘kWh/MT of clinker’

indicates:

- Energy Index Parameter

- Utility Factor

- Production Factor

- Load Factor

d. An energy policy does not include:

- Target energy consumption reduction

- Time period for reduction

- Declaration of top management commitment

- Future production projection

P.T.O.

e. The tool used for performance assessment and logical

evaluation of avenues for improvement in energy management

and audit is

- Fuel substitution

- Monitoring and verification

- Energy pricing

- Bench marking

f. The empirical relationship used to plot production Vs Energy

consumption is (Y= energy consumed for the period; C = fixed

energy consumption; M = energy consumption directly related

to production; X= production for same period).

- X=Y+MC

- Y=Mx+C

- M=Cx+Y

- X=MY+C

g. The term ‘energy services contract’ in a project comes under

- Financing

- contracting

- monitoring

- evaluation

h. Project management technique which uses three time estimates-

optimistic, pessimistic and most likely, which help in

establishing the probability of completing a project within a

specified time and take calculated risk before commencing a

project is

- PERT

- CUSUM

- CPM

- Gantt Chart

I. The support for energy management is expressed in a formal

written declaration of commitment. This is called

- Company policy

- Management policy

- Energy policy

- Energy efficiency policy

j. Amount of oxygen (stoichiometric) needed for complete

combustion of 1 kg of Methane

- 17.39 kg

- 4 kg

- 6 kg

- 2 kg

2. Answer the following shortly:                                         3x5=15

- Explain: Energy management, energy conservation and energy auditing.

- Why the economic analysis is so important in energy management programme?

- What are the base line data that an audit team should collect while conducting detailed energy audit?

- Name the various sources in the plant from where information related to energy use data can be obtained?

- What do you understand about ESCOs?

3. Answer the following in detail:                               4+3+4+4=15

- Explain the use of Sankey diagram. How material and energy balance helps in energy conservation?

- Explain the limitations with ‘Simple Payback Period’ technique with an example.

- For complete combustion of 1 kg of a typical coal is required 12 kg of air. Calorific value of coal is 4200 kCal/kg with ash content of 22%. What is the quantity (in kg) of flue gas generated by burning 5 kg coal?

- A cement industry has installed 6 MW DG set and is operated by furnace oil. The cost of furnace oil is Rs 35/liter. The average loading on the DG set is 5 MW and the hourly furnace oil consumption is 1230 liters. Estimate the cost of energy generation in Rs/kWh.

P.T.O.