TU/CODL

TEZPUR UNIVERSITY

SEMESTER END EXAMINATION (SPRING) 2020

DRE 104: WIND AND HYDRO ENERGY

Full Marks: 70		           Time: 3 hours

The figures in the right-hand margin indicate marks for the individual questions

***************************************************

1. Choose the right answer-                                                          2×10=20

- Which of the following energy source is an indirect method of solar energy utilization?

- Wind Energy

- Geothermal Energy

- Nuclear energy

- Tidal Energy

- The instrument preferred in the measurement of highly fluctuating velocities in air flows is-

- Hot Wire Anemometer

- Propeller type Anemometer

- Pitot Static Tube

- Three cup Anemometer

- Where is the largest Wind Farm located in India?

- Jaisalmer, Rajasthan

- Muppandal, Tamil Nadu

- Vaspet, Maharashtra

- Chakala, Maharashtra

- The unit of scale parameter in Weibull distribution is-

- No unit

- m2/s

- m/s

- kg/m3

- The coefficient of performance of a wind turbine for maximum power output is-

- 13/18

- 11/17

- 5/8

- 16/27

P.T.O.

- In hydro power plants-

- Initial cost is high and operating cost is low

- Initial cost as well as operating costs are high

- Initial cost is low and operating cost is high

- Initial cost as well as operating costs are low

- The share of hydro power in India’s electricity installed capacity is-

- 13%

- 26%

- 23%

- 45%

- The installed capacity of Three Gorges dam of china is

- 22.5 GW

- 18 GW

- 32.5 GW

- 12.5 GW

- A Kaplan turbine is

- a high head mixed flow turbine

- an impulse turbine, inward flow type

- a reaction turbine, outward flow type

- low head axial flow turbine

- The terms Darrious rotor and Savonius rotor are related to

- Pico hydropower

- Vertical axis wind turbine

- Horizontal axis wind turbine

- Battery charging systems

2. Answer the following questions:                                         5+5+5=15

- What is the maximum energy available from the wind? Explain the dependency of various parameters on the power output.

- Why site specifications are so important for wind energy applications? How do you represent the wind data statistically?

- Define Betz limit and derive the value for CP (max).

3. Answer the following questions:                                    4+2+3+6=15

- Define: Airfoil; tip speed ratio, pitch regulation, capacity factor

- Write down the assumptions used in axial momentum theory and blade element theory.

- What are the main differences between wind mill and wind electric generator?

- Consider a wind turbine with 5 m diameter rotor. Speed of the rotor at 8 m/s wind velocity is 100 r/min and its power coefficient at this point is 0.32. Calculate the tip speed ratio and torque coefficient of the turbine. Assume the density of air is 1.12 kg/m3.

4. Answer the following questions:                               4+3+3+4+6=20

- Draw a schematic diagram of a hydropower plant. Explain in brief the various components of the power plant.

- Describe the operation of a pumped hydro storage plant.

- Classify the hydro-turbines according to head, power and specific speed.

- Define: unit speed, unit power and unit discharge and derive their relevant relations.

- The following data refers to a proposed hydroelectric power plant. Available head =27m, catchment area 430 km2, Rainfall = 150 cm/year, Penstock efficiency = 95%, turbine efficiency = 80%, generator efficiency =86% and load factor =0.45. Calculate the power developed and suggest suitable turbines for the plant.

6

***