TU/CDOE

TEZPUR UNIVERSITY

SEMESTER END EXAMINATION (SPRING) 2021

DRE 202: ENERGY EFFICIENCY IN THERMAL UTILITIES

Total Marks: 70                                                       Time: 3 Hours

Figures in the margin indicate full marks for the questions

All questions are compulsory

-----------------------------------------------------------------------------------

1. Fill in the blanks.	          		                                       1×10=10

- The basic cycle of operation of the gas power plants is known as the _________ cycle.

- Critical boilers operate at pressures above __________ bar.

- Heat transfer may be defined as the transfer of energy from one region to another due to __________ gradient.

- The R-value gives the effectiveness of __________ materials. It is basically the thermal resistance of the material per unit surface area.

- Steam traps operating due to the difference in temperature between steam and cooled static condensate are known as __________.

- Discharging of water from boiler in order to remove the suspended solids and bottom sludge from the boilers is known as __________.

- The temperature at which a liquid fuel stops flowing is known as the __________ point.

- __________ systems are designed to recover the unused heat to do some useful work.

- In heat exchanger analysis, the product of the mass flow rate and the specific heat of a fluid are combined into a single quantity called the __________.

- The __________ diagram is used to visualise the magnitude of flow, between different nodes in a network, through the width of the arrows.

2.  Answer the following :                                                                  5+5= 10

- A coal fired boiler converts 2 kg of water at 25°C into dry steam at 5 bar. How much heat generated by the combustion of the fuel is used in the conversion of water to dry steam? (Given hf = 640.1 kJ/kg and hfg  = 2107.4 kJ/kg at 5 bar)

P.T.O.

TU/CDOE

TEZPUR UNIVERSITY

SEMESTER END EXAMINATION (SPRING) 2021

DRE 202: ENERGY EFFICIENCY IN THERMAL UTILITIES

Total Marks: 70                                                       Time: 3 Hours

Figures in the margin indicate full marks for the questions

All questions are compulsory

-----------------------------------------------------------------------------------

1. Fill in the blanks.	          		                                       1×10=10

- The basic cycle of operation of the gas power plants is known as the _________ cycle.

- Critical boilers operate at pressures above __________ bar.

- Heat transfer may be defined as the transfer of energy from one region to another due to __________ gradient.

- The R-value gives the effectiveness of __________ materials. It is basically the thermal resistance of the material per unit surface area.

- Steam traps operating due to the difference in temperature between steam and cooled static condensate are known as __________.

- Discharging of water from boiler in order to remove the suspended solids and bottom sludge from the boilers is known as __________.

- The temperature at which a liquid fuel stops flowing is known as the __________ point.

- __________ systems are designed to recover the unused heat to do some useful work.

- In heat exchanger analysis, the product of the mass flow rate and the specific heat of a fluid are combined into a single quantity called the __________.

- The __________ diagram is used to visualise the magnitude of flow, between different nodes in a network, through the width of the arrows.

2.  Answer the following :                                                                  5+5= 10

- A coal fired boiler converts 2 kg of water at 25°C into dry steam at 5 bar. How much heat generated by the combustion of the fuel is used in the conversion of water to dry steam? (Given hf = 640.1 kJ/kg and hfg  = 2107.4 kJ/kg at 5 bar)

P.T.O.

- Do you think that the properties of coal influence the performance of the equipment utilizing it? Justify your answer highlighting the relevant properties of coal.

3. What are the functions of a blowdown valve? How do you select diameter of a steam pipe? Mention different types of steam traps used in thermal power plant?                                                    3+4+3 = 10

4. Are topping cycle cogeneration systems more effective than bottoming cycle cogeneration system? Justify your answer with the help of thermodynamic consideration. Also explain various parameters associated with the performance evaluation of cogeneration systems.                                                                       5+5 = 10

5. State and discuss five different cases of waste heat recovery. Also explain the consequences if waste heat is not recovered.      5+5 = 10

6. Explain the important properties of insulating materials. Also discuss about the insulating materials recommended for normal temperature, high temperature and very high temperature applications. 		                                                                       5+5= 10

7. Water at the rate of 20kg/minute is heated from 35°C to 75°C by oil having a specific heat of 2 kJ/kg/°C. The fluids are used in a counter-flow heat exchanger and oil enters the heat exchanger at 110°C and leaves at 75°C. The overall heat transfer coefficient is 320W/m2/°C. Calculate the heat exchanger area.                                   5

8. A 30 mm outer diameter pipe is to be covered with two layers of insulation each having a thickness of 25 mm. The average thermal conductivity of one insulating material is 5 times that of the other. Determine the percentage decrease in heat transfer, if the better insulating material is put next to the pipe than when it forms the outer layer. Assume that the outside and inside surface temperatures of the composite insulation are fixed.                              5

***

- Do you think that the properties of coal influence the performance of the equipment utilizing it? Justify your answer highlighting the relevant properties of coal.

3. What are the functions of a blowdown valve? How do you select diameter of a steam pipe? Mention different types of steam traps used in thermal power plant?                                                    3+4+3 = 10

4. Are topping cycle cogeneration systems more effective than bottoming cycle cogeneration system? Justify your answer with the help of thermodynamic consideration. Also explain various parameters associated with the performance evaluation of cogeneration systems.                                                                       5+5 = 10

5. State and discuss five different cases of waste heat recovery. Also explain the consequences if waste heat is not recovered.      5+5 = 10

6. Explain the important properties of insulating materials. Also discuss about the insulating materials recommended for normal temperature, high temperature and very high temperature applications. 		                                                                       5+5= 10

7. Water at the rate of 20kg/minute is heated from 35°C to 75°C by oil having a specific heat of 2 kJ/kg/°C. The fluids are used in a counter-flow heat exchanger and oil enters the heat exchanger at 110°C and leaves at 75°C. The overall heat transfer coefficient is 320W/m2/°C. Calculate the heat exchanger area.                                   5

8. A 30 mm outer diameter pipe is to be covered with two layers of insulation each having a thickness of 25 mm. The average thermal conductivity of one insulating material is 5 times that of the other. Determine the percentage decrease in heat transfer, if the better insulating material is put next to the pipe than when it forms the outer layer. Assume that the outside and inside surface temperatures of the composite insulation are fixed.                              5

***